## Introduce Front Panel

Modern > Numeric > Dial

Modern > Graph > Waveform Graph

Modern > Boolean > Stop Button

Modern > Boolean > Stop Button

## Introduce Build Diagram

Express > Input > Simulate Signal

Programming > Structures > While Loop Button

Programming > Structures > While Loop Button

Express > Input > Simulate Signal

Change Samples per Second to

48000

Change Number of Samples to

12000

Uncheck Automatic

## Go to Front Panel and run the VI by clicking the Run Arrow

Run Arrow

## Add a Loop Delay

Express > Exec Control > Time Delay Button

## Save you VI to a place that is easy to remember called it “Exercise 1”

## Add a 2nd Signal Generator and Control with a Copy and Paste – Add Waveforms together and rewire

Programming > Numeric > Add

- Add Play Waveform Express VI from the Output Folder of Function Palette

Express > Output > Play Waveform

- Wire to 2 tone waveform wire

## On the Front Panel add Digital Display to both Knobs by right clicking

## Create a SubVI (subroutine)

## Open Front Panel of new subVI, run it by itself, save it as “Play Two Tones”

DTMF keypad frequencies

|  | 1209 Hz | 1336 Hz | 1477 Hz |
| --- | --- | --- | --- |
| 697 Hz | 1 | 2 | 3 |
| 770 Hz | 4 | 5 | 6 |
| 852 Hz | 7 | 8 | 9 |
| 941 Hz | * | 0 | # |

| Event | Low frequency | High frequency |
| --- | --- | --- |
| Busy signal | 480 Hz | 620 Hz |
| Dial tone | 350 Hz | 440 Hz |
| Ringback tone(US) | 440 Hz | 480 Hz |

## Edit Icon of  “Play Two Tones” VI

Right Click Here & select Edit Icon

## Create 2 numeric arrays on the Front Panel and wire them into a For Loop in the block diagram

Programming> Structure > For Loop

Modern > Array, Matrix &Cluster > Array

Modern > Numeric  > Numeric Control

Select a VI > browse to SubVI called “Play Two Tones”

Right Click Here & select Disable Indexing

Programming > Numeric > Conversion> To Word Integer

Right Click Here & select Disable Indexing

Programming > Numeric > Square Root

Programming > Numeric > Square Root

Programming > Array > Index Array

Right Click Here & select Create > Indicator

Use “Make current values Default” to save values in the two Array Controls – Go to Menu Bar > Edit

Save as “Exercise 1.5”

## Add Hardware Input (Joystick) with Case Structure

  - Search Function Palette for “Joystick”

  - Document your code

  - Add to Delay to the loop

  - Add “Unbundle by name” to Button Info Wire

  - Add “Unbundle by name” to Axis Info Wire

  - Resize the “Unbundle by name” to have to output

  - Add a Case Structure

  - Optional  - Add a Strip Chart to the Front Panel

Connectivity > Input Device Control > …

Select a VI …

Programming > Structures >Case Structure

Programming > Timing > Wait Until next ms multiple

Programming > Cluster, Class & Variant > Unbundle by Name

Programming > Numeric >Divide

Optional

Add some Boolean algebra

Or

Comparison Functions

# Exercise 3

Enumerated Data Type

Case Structure

Shift Register

Modern  > Ring & Enum > Enum

Optional

Probes can be used

# Exercise 4

Local and Global Variables

# LabVIEW Cheat Sheet

## Common Mistakes

Can’t find all the Libraries on the Functions Palette

- Is the Block Diagram Window Open?  If not – Menu bar >Window > Show Block Diagram	<ctrl> E

- Do you need to switch the Palette Set? – Menu bar >Tools > Switch Palette Set		No Short Cut

The tool does not change to different modes

- You turned off Auto Tool Mode – Hit <shift> <tab>

- An object or objects is selected – Left Click in a Free Space

- The VI is running – you can’t edit a VI when it is running – stop the VI

Broken Wires

- You wired data types that don’t match

The While Loop does not loop or only runs once

Check the conditional terminal of While Loop; you have accidentally switched “Stop if True” to  “Continue if True” or vice versa.

When you place new Icons on the Block Diagram, they are automatically wired together and you want to stop this

Turn off Autowiring, go to the Menu Bar > Tools > Options > Block Diagram – uncheck Enable Auto Wire

When you are entering or editing text, it creates a new line of text

Click the “Enter Text” icon and this will not create a new line of text. Hitting the <Enter >  key will create a new line of text.

## Most Common Shortcuts

<Ctrl> E			Switch between Front Panel and Block Diagram

<Ctrl> Z			Undo

<Shift> <Ctrl> Z		Redo

<Ctrl> C			Copy

<Ctrl> X			Cut

<Ctrl> V			Paste

<Ctrl> B			Remove Bad Wires

<Ctrl> S			Save

<Ctrl> Left Click		Copy and Drag

Right Click on objects on the Front Panel and Block Diagram to open an object specific Menu